Service Manual

LCD Monitor S7G 17

Service Manual Versions and Revision

| No. | Version | Release Date | Revision |
| --- | --- | --- | --- |
| 1 | 1.0 | 2006/03/27 | Initial Release |

Copyright

Copyright 2005 Innolux Display Corp. Ltd

All Rights Reserved

This manual may not, in whole or in part, be copied, Photocopied, reproduced, translated, or converted to any electronic or machine readable form without prior written permission of Innolux Display Corp. Ltd.

S7- 17 Service Manual

Table of Contents

Chapter 1- PRECAUTIONS & SAFETY NOTICES	3

1.	SAFETY PRECAUTIONS	3

2.	PRODUCT SAFETY NOTICE	3

3.	SERVICE NOTES	3

Chapter 2- SERVICE TOOLS & EQUIPMENT REQUIRED	4

Chapter 3- CIRCUIT THEORY	5

1.	Block Diagram	5

2.	Electronic Circuit Theory	6

3.	SUPPORT TIMING TABLE	8

4.	D-SUB CONNECTOR PIN ASSIGNMENT	8

5.	CN102(LVDS INTERFACE)	8

6.	Key Parts Pin Assignments	9

Chapter 4- Disassembly & Assembly	13

1.	Exploded Diagram	13

2.	Disassembly Block	14

3.	Assembly Block	15

Chapter 5- TEST AND ADJUSTMENT	16

1.	Function key Definitions	16

2.	OSD Control	16

3.	Factory Mode Introduction	17

4.	Burn-in pattern	18

5.	Auto Color Balance	18

6.	EDID (Rewrite EDID data to EEPROM)	18

7.	Upload firmware to MCU via VGA Cable	18

8.	After repair, to ensure the quality you should do the following test and adjustment.  .	18

Chapter 6- TROUBLE SHOOTING	20

1.	Inverter trouble shooting	20

2.	POWER TROUBLE SHOOTING-1	21

3.	POWER TROUBLE SHOOTING-2	22

4.	POWER TROUBLE SHOOTING-3	23

5.	Black Screen	24

6.	White Screen	25

7.	Bad Screen	26

Chapter 7- RECOMMENDED PART LIST	27

Attachment 1- Schematic	33

Attachment 2- PCB Layout	40

# Chapter 1- PRECAUTIONS & SAFETY NOTICES

## 1. SAFETY PRECAUTIONS

This monitor is manufactured and tested on a ground principle that a user’s safety comes first. However, improper used or installation may cause damage to the monitor as well as to the user.

WARNINGS:

- This monitor should be operated only at the correct power sources indicated on the label on the rear of the monitor. If you’re unsure of the power supply in you residence, consult your local dealer or Power Company.

- Do not try to repair the monitor by yourself, as it contains no user-serviceable parts. This monitor should only be repaired by a qualified technician.

- Do not remove the monitor cabinet. There is high-voltage parts inside that may cause electric shock to human bodies.

- Stop using the monitor if the cabinet is damaged. Have it checked by a service technician.

- Put your monitor only in a lean, cool, dry environment. If it gets wet, unplug the power cable immediately and consult your closed dealer.

- Always unplug the monitor before cleaning it. Clean the cabinet with a clean, dry cloth. Apply non-ammonia based cleaner onto the cloth, not directly onto the class screen.

- Do not place heavy objects on the monitor or power cord.

## 2. PRODUCT SAFETY NOTICE

Many electrical and mechanical parts in this chassis have special safety visual inspections and the protection afforded by them cannot necessarily be obtained by using replacement components rated for higher voltage, wattage, etc. Before replacing any of these components read the parts list in this manual carefully. The use of substitute replacement parts, which do not have the same safety characteristics as specified in the parts list, may create shock, fire, or other hazards.

## 3. SERVICE NOTES

- When replacing parts or circuit boards, clamp the lead wires around terminals before soldering.

- Keep wires away from high voltage, high temperature components and sharp edges.

- Keep wires in their original position so as to reduce interference.

- Adjustment of this product please refers to the user’ manual.

# Chapter 2- SERVICE TOOLS & EQUIPMENT REQUIRED

1.  VIDEO TEST SIGNAL GENERATOR

2.  AUDIO TEST SIGNAL GENERATOR

3.  MULTIMETER

4.  SCREW DRIVER

5.  OSCILLOSCOPE

6.  IRON

7.  SOLDER

8.  VGA CABLE (15PINS POINT TO POINT)

9. 	AUDIO CABLE

10. 	COLOR ANALYZER

11. 	ISP BOARD

12.	EDID BOARD

13.	FOXISP. EXE FILE

14.	EDID PROGRAM FILE

Chapter 3- CIRCUIT THEORY

## 1. Block Diagram

EMI Filter Circuit

5V Switch to Panel

Power Switch transformer

FS8860-18CJ3.3V to 1.8V

MCU (MTV512MN)

Inverter Circuit  OZ9910G

Rectifier filter 5V Output

Photocoupler

Protection feedback

EEPROM

24C02

Key&LED

Sampling Voltage

EEPROM

24C16

Audio Input

AC Input

VGA Input

Key Input

Speaker

Panel

Back lamp

FS8860-33CJ 5V to 3.3V

Rectifier& Filter Circuit

Rectifier filter 12V Output

5V Switch to inverter

Sampling Voltage feedback

VGA Input Circuit

SCALER

(TSU16AS)

Audio Amplifier

(TDA7053)

MCU

Volume Control

5V Input

B

ON/OFF

ON/OFF

MOSFET Switch PWM Control Circuit TOP245Y

## 2. Electronic Circuit Theory

2.1  Switching Mode Power Supply

2.1.1 AC Current Input Circuit

P801 is a connector for connecting AC Power. F801 is a fuse to protect all the circuit. AC input voltage is from 90V to 264V. R820 and R821 joined between two inputting main circuit to prevent man from shock. L801 is used to clear up low frequency wave. C801 and C806 are used to discharge the waves that L801 produced. High frequency waves are damped by C801 and C806. D801 is a rectifier which composed of 4 build-in diodes, it inverts AC to DC.

    - High Voltage to Low Voltage Control Circuit

C805 is used to smooth the wave from rectifier. IC802 is a highly integrated PWM controller, which build-in a power MOSFET. When rectified DC high voltage is applied to the DRAIN pin during start-up, the MOSFET is off initially, and the CONTROL pin capacitor is charged through a switched high voltage current source connected internally between the DRAIN and CONTROL pins. When the CONTROL pin voltage Vc reaches approximately 5.8V, the control circuitry is activated and the soft-start begins. The soft-start circuit gradually increases the duty cycle of the MOSFET from zero to the maximum value over approximately 10ms. If no external feedback/supply current is fed into the CONTROL pin by the end of the soft-start, the high voltage current source is turned off and the CONTROL pin will start discharging in response to the supply current drawn by the control circuitry.

Resistor R803, R807, R824 and R825 are for line over voltage shut-down (OVP) and line under-voltage detection (UVP).Resistors R801, R805, R822, and R823 are for external current limit adjustment, and used to reduce the current limit externally to a value close to the operating peak current of primary about 1.35A. The mean is power will protected when the primary current over about 1.35A.

When PWM is turned off, the main current flow will be consumed through D804 and ZD802, This will prevent MOSFET which built-in IC802 from being damaged under large current impulse and voltage spike.

D806 and C815 provide internal Auxiliary current to CONTROL pin during normal operation. In addition, error amplifier and feedback current to the CONTROL pin are for duty cycle control.

    - DC_5V and DC_12V Output Circuit

For DC 5V, D805 is used to rectify the inducted current. R806 and C811 are used to store energy when current is reversed. The parts including C812, C814, C822, C821, B801 and L803 are used to smooth the current waves.

For DC 12V, D803 is used to rectify the inducted current. R802 and C802 are used to store energy when current is reversed. The parts including C808, C810 and L802 are used to smooth the current waves.

    - Feedback and OVP Protect Circuit

Pin R of IC803 is supplied 2.5V stable voltage. It is connected to 5V and 12V output through R811, R810 and R818. R811, R810 and R818 are output sampling resistor. When the sampling voltage more than 2.5V or less than 2.5V, feedback current of IC802 will change, this can change the voltage from transformer T801.

For 5VDC output OVP, ZD803 is a zener diode, when 5V output voltage becomes up to 5.6V, the zener current cause R819 voltage become up to 0.7V, Q801 is triggered and OVP starts. For 12VDC output OVP, ZD804 is a Zener Diode, when 12V output voltage becomes up to 15V, the zener current cause R819 voltage become up to 0.7V, Q801 is triggered and OVP starts. The collector current of Q801 is used to make build-in diode light. FB Current of IC802 will be changed; it can change the voltage from T801.

Q802, R827, R828 and ZD801 make up of dummy loading circuit. For start-up sequence, during 5V output take place high loading first, this dummy loading circuit operated to insure 12V not be increased

2.2	Inverter Circuit

2.2.1	Low voltage to high voltage circuit

12VDC provides the power for IC501; the control signals Brightness and ON/OFF come from I/F board. ON/OFF signal connect to pin8 of IC501 and makes IC501 enable. Brightness signal connect to pin7 of IC501 and regulates the panel brightness, R524, R529, C505 make up a network of delaying time circuit and R528, R523, R524 make up a divided voltage network, C504 is used to dump noise. The operation frequency is determined by the external Resistor R522 and capacitor C529 connected to pin5 of IC501. BURST MODE regulated dimming frequency is determined by the external resistor R527 and capacitor C506 connected to pin6 of IC501. C502 is used for soft start and compensation, C502, C528 are used for dump noise.

The output drives, include NDR4, NDRV2, PDRV3, PDRV1 (pins1, 3, 15, 16 respectively) output square pulses to drive MOSFET U501, U502, and each of U501, U502 is consist of a N channel MOSFET and a P channel MOSFET. U501 and U502 work as full-bridge topology, it is high efficient, zero voltage switching.

During start up, VSEN (pin9) senses the voltage at the transformer secondary. When VSEN reaches 3.0V, the output voltage is regulated. If no current is sensed approximately 1.5 seconds IC501 shunt off.

The current flowing through CCFL is sensed and regulated through sense resistor R509, R511. The feedback voltage through R506, R507, C508 connected to Pin11 (ISEN), then compared with a reference voltage (1.5V) via a current amplifier, resulting in PWM drive outputs to full-bridge switches.

2.2.2	Protection circuit

Over Voltage Protection: R501and R502 are connected in high voltage output connector, the divided AC voltage is inverted DC voltage through D508, R505 and C507are used to rectify wave & dump noise. Then the voltage signal reaches Pin9 VSEN of IC501, when the voltage changes, build-in PWM of IC501 will adjust output voltage.

Open Lamp Protection: In normal operation, the resistors R510, R511, R512, R509 are sensed a high level AC voltage, the AC signal IS1 invert DC voltage through D509, R515, C533, and the high level DC voltage reaches the gate pin of Q502, similarly, the gate pin of Q503, Q504, Q505 has high level DC voltage. So the gate pin of Q501 has a low level voltage, and the IC501 is normal operation.  Once one of signal IS1, IS2, IS3, and IS4 is low, the voltages of Q501 gate pin is high level, and make the voltage of ISEN low level, the IC501 will be shunt down.

2.3	I/F Circuit

2.3.1	Power Supply

- +5V is converted to 3.3V and 1.8V for Scaler, 3.3V to MCU.

- +5V is for LVDS panel via Q103.

- AU_VCC +5V is for audio amp.

2.3.2	Control Signals Output

- ON/SLEEP at U104#2 via Q102 reversed to CCFL_enable to control backlight on/off. It keeps a high voltage level to make the backlight on.

- DIMMER at U103#28 output to control backlight level.

2.3.3	RGB Input

- Signal RED,GREEN,BLUE input through CN101 #1,#2,#3, Stop DC via C122, C123 and C124, then enter into U103 (TSU16AS) analog input terminal #19,#16,#14, and then TSU16AS deals with signal internally. BAV99 ( TVS101/102/103) are ESD protector to prevent Scaler from ESD.R113/114/115 and R116/117/118 are matching resistance.

- Signal DDC_CLK (series clock) inputs via CN101#15, and then passes through TVS104 for over voltage protection, goes into EDID EEPROM IC U102 #6.

- Signal DDC_DAT (series data) inputs via CN101#12, and then passes through TVS106 for over

voltage protection, goes into EDID EEPROM IC U102 #5.

- Signal TTL vertical sync. (Vsync) inputs via CN101 #14, and then clamped by ZD104 (5.6V) Zener, passes through R120, and then goes into IC U103 (TSU16AS) #9.

- Signal TTL horizontal sync. (Hsync) inputs via CN101 #13, and then clamped byZD103 (5.6V) Zener, passes through U107 and then goes into IC U103 (TSU16AS) #8.

- CN101 #5 is defined as VGA cable detect pin, this detector realize via R119 to U104 #3, When cable plug in, VGA_PLUG should be pulled low.

- U102 VCC is supplied by 5V_VGA from PC via CN101#9 or by power circuit +5V. Both can get across to U102 #8 with D101.

- U102 is an EEPROM IC which is memory and EDID data saved in it.

2.3.4	Audio Circuit

-U701 is a stereo audio power amplifier IC (TDA7053).

-L/R audio signal input via C703/C704 into U701 Pin4/6, Audio power amplifier signal from U701 Pin9/12 and Pin13/16 output To speaker (1W/8Ω x2).

-U701 Pin5 is supply voltage (+5V) terminal. Pin2/8 terminal for DC volume control, From MCU Pin4 output volume control signal via R706 to Control U701 Pin2/8 volume terminal, DC voltage range is 0 to 5V.

## 3. Support Timing Table

| VESA 640 x 480 @ 60 Hz | VESA 1024 x 768 @ 70 Hz |
| --- | --- |
| VESA 640 x 480 @ 75 Hz | VESA 1024 x 768 @ 75 Hz |
| VESA 800 x 600 @ 56 Hz | VESA 1280 x 1024 @60Hz |
| VESA 800 x 600 @ 75 Hz | VESA 1280 x 1024 @ 75Hz |
| VESA 1024 x 768 @ 60 Hz | 720 x 400 @ 70Hz |

## 4. D-SUB Connector Pin Assignment

| Pin | Symbol | Pin | Symbol | Pin | Symbol |
| --- | --- | --- | --- | --- | --- |
| 1 | Red+ | 6 | Red_GND | 11 | GND |
| 2 | Green+ | 7 | Green_GND | 12 | DDC_DAT |
| 3 | Blue+ | 8 | Blue_GND | 13 | Hsync |
| 4 | NC | 9 | 5V_VGA | 14 | Vsync |
| 5 | Cable Detect | 10 | GND | 15 | DDC_CLK |

## 5. CN102 (LVDS interface)

| Pin No | Symbol | Description |  | Pin No | Symbol |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | RxO0- | LVDS Differential data input Channel 0(-) |  | 16 |  | RxE1+ |  | LVDS Differential data input Channel 1(+) |  |
| 2 | RxO0+ | LVDS Differential data input Channel 0(+) |  | 17 |  | GND |  | Ground |  |
| 3 | RxO1- | LVDS Differential data input Channel 1(-) |  | 18 |  | RxE2- |  | LVDS Differential data input Channel 2(-) |  |
| 4 | RxO1+ | LVDS Differential data input Channel 1(+) |  | 19 |  | RxE2+ |  | LVDS Differential data input Channel 2(+) |  |
| 5 | RxO2- | LVDS Differential data input Channel 2(-) |  | 20 |  | RxEC- |  | LVDS Differential Clock input (-) |  |
| 6 | RxO2+ | LVDS Differential data input Channel 2(+) |  | 21 |  | RxEC+ |  | LVDS Differential Clock input (+) |  |
| 7 | GND | Ground |  | 22 |  | RxE3- |  | LVDS Differential data input Channel 3(-) |  |
| 8 | RxOC- | LVDS Differential Clock input (-) |  | 23 |  | RxE3+ |  | LVDS Differential data input Channel 3(+) |  |
| 9 | RxOC+ | LVDS Differential Clock input (+) |  | 24 |  | GND |  | Ground |  |
| 10 | RxO3- | LVDS Differential data input Channel 3(-) |  | 25 |  | GND |  | Ground |  |
| 11 | RxO3+ | LVDS Differential data input Channel 3(+) |  | 26 |  | GND |  | Ground or Open |  |
| 12 | RxE0- | LVDS Differential data input Channel 0(-) |  | 27 |  | GND |  | Ground |  |
| 13 | RxE0+ | LVDS Differential data input Channel 0(+) |  | 28 |  | VCC |  | Power supply (+5.0V) |  |
| 14 | GND | Ground |  | 29 |  | VCC |  | Power supply (+5.0V) |  |
| 15 | RxE1- | LVDS Differential data input Channel 1(-) |  | 30 |  | VCC |  | Power supply (+5.0V) |  |

## 6. Key Parts Pin Assignments

6.1 U103 (Scaler IC TSU16AS) Pin description

| 6.11 CPU Interface |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Pin Name | Pin Type | Function |  | Pin |  |
| HWRESET | Schmitt Trigger Input w/ 5V-tolerant | Hardware reset; active high |  | 5 |  |
| INT | Output | CPU | interrupt; 4mA driving strength | 27 |  |
| AD3 | I/O w/ 5V-tolerant | DDR direct bus AD3; 4mA driving strength |  | 4 |  |
| AD2 | I/O w/ 5V-tolerant | DDR direct bus AD2; 4mA driving strength |  | 1 |  |
| AD1 | I/O w/ 5V-tolerant | DDR direct bus AD1; 4mA driving strength |  | 2 |  |
| AD0 | I/O w/ 5V-tolerant | DDR direct bus AD0; 4mA driving strength |  | 3 |  |
| ALE | I w/ 5V-tolerant | DDR direct bus ALE; active high |  | 24 |  |
| WRZ | I w/ 5V-tolerant | DDR direct bus WRZ; active low |  | 25 |  |
| RDZ | I w/ 5V-tolerant | DDR direct bus RDZ; active low |  | 26 |  |

6.12 Analog Interface

| Pin Name | Pin Type |  | Function |  |  |  |  |  |  |  |  |  |  | Pin |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HSYNC | Schmitt Trigger Input w/ 5V-tolerant |  | Analog HSYNC |  |  |  |  | input |  |  |  |  |  | 8 |  |
| VSYNC | Schmitt Trigger Input w/ 5V-tolerant |  | Analog VSYNC input |  |  |  |  |  |  |  |  |  |  | 9 |  |
| REFP |  |  | Internal |  | ADC top de-coupling pin |  |  |  |  |  |  |  |  | 22 |  |
| REFM |  |  | Internal |  | ADC bottom de-coupling pin |  |  |  |  |  |  |  |  | 23 |  |
| RINP | Analog Input |  | Analog red input |  |  |  |  |  |  |  |  |  |  | 19 |  |
| RINM | Analog Input |  | Reference ground for analog red input |  |  |  |  |  |  |  |  |  |  | 18 |  |
| SOGIN | Analog Input |  | Sync-on-green |  |  |  |  | input |  |  |  |  |  | 17 |  |
| GINP | Analog Input |  | Analog green |  |  |  | input |  |  |  |  |  |  | 16 |  |
| GINM | Analog Input |  | Reference ground for analog green |  |  |  |  |  |  |  |  | input |  | 15 |  |
| BINP | Analog Input |  | Analog blue |  |  | input |  |  |  |  |  |  |  | 14 |  |
| BINM | Analog Input |  | Reference ground for analog blue |  |  |  |  |  |  |  | input |  |  | 13 |  |
| REXT |  |  | External resistor 390 ohm to AVDD_ADC |  |  |  |  |  |  |  |  |  |  | 11 |  |
| 6.13 LVDS Interface |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pin Name |  | Pin Type |  | Function |  |  |  |  |  |  |  |  | Pin |  |  |
| LVA0M |  | Output |  | A-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 42 |  |  |
| LVA0P |  | Output |  | A-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 41 |  |  |
| LVA1M |  | Output |  | A-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 40 |  |  |
| LVA1P |  | Output |  | A-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 39 |  |  |
| LVA2M |  | Output |  | A-Link Negative LVDS Differential Data Output |  |  |  |  |  |  |  |  | 38 |  |  |
| LVA2P |  | Output |  | A-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 37 |  |  |
| LVA3M |  | Output |  | A-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 34 |  |  |
| LVA3P |  | Output |  | A-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 33 |  |  |
| LVACKM |  | Output |  | A-Link Negative LVDS Differential Clock Output |  |  |  |  |  |  |  |  | 36 |  |  |
| LVACKP |  | Output |  | A-Link Positive LVDS Differential Clock Output |  |  |  |  |  |  |  |  | 35 |  |  |
| LVB0M |  | Output |  | B-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 52 |  |  |
| LVB0P |  | Output |  | B-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 51 |  |  |
| LVB1M |  | Output |  | B-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 50 |  |  |
| LVB1P |  | Output |  | B-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 49 |  |  |
| LVB2M |  | Output |  | B-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 48 |  |  |
| LVB2P |  | Output |  | B-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 47 |  |  |
| LVB3M |  | Output |  | B-Link Negative LVDS Differential |  |  |  |  |  | Data Output |  |  | 44 |  |  |
| LVB3P |  | Output |  | B-Link Positive LVDS Differential |  |  |  |  | Data Output |  |  |  | 43 |  |  |
| LVBCKM |  | Output |  | B-Link Negative LVDS Differential Clock Output |  |  |  |  |  |  |  |  | 46 |  |  |
| LVBCKP |  | Output |  | B-Link Positive LVDS Differential Clock Output |  |  |  |  |  |  |  |  | 45 |  |  |

| 6.14 GPO Interface |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pin Name | Pin Type |  |  | Function |  |  | Pin |  |
| PWM1 | Output |  |  | PWM1; 4mA driving strength |  |  | 29 |  |
| PWM0 | Output |  |  | PWM0; 4mA driving strength |  |  | 28 |  |
| 6.15 Misc. Interface |  |  |  |  |  |  |  |  |
| Pin Name | Pin Type |  |  | Function |  |  | Pin |  |
| XIN | Crystal | Oscillator Input |  | Xin |  |  | 6 |  |
| XOUT | Crystal | Oscillator Output |  | Xout |  |  | 7 |  |
| VCTRL | Output |  |  | Regulator Control |  |  | 54 |  |
| 6.16 Power Pins |  |  |  |  |  |  |  |  |
| Pin Name | Pin Type |  |  | Function |  | Pin |  |  |
| AVDD_ADC | 3.3V Power |  |  | ADC Power |  | 12, 20 |  |  |
| VDDP | 3.3V Power |  |  | Digital | Output Power | 30, 53 |  |  |
| VDDC | 1.8V Power |  |  | Digital Core Power |  | 32, 55 |  |  |
| GND | Ground |  |  | Ground |  | 10, 21, 31, 56 |  |  |

  - U104 (MCU MTV512) Pin description (CS8954)

| Name | Pin No. | I/O | Description |  |
| --- | --- | --- | --- | --- |
| P5.0/DA0 | 1 | I/O | General purpose I/O | (Open drain) |
| P5.1/DA1 | 2 | I/O | General purpose I/O | (Open drain) |
| P5.2/DA2 | 3 | I/O | General purpose I/O | (Open drain) |
| P5.3/DA3 | 4 | I/O | General purpose I/O | (Open drain) |
| P5.4/DA4 | 5 | I/O | General purpose I/O | (Open drain) |
| P5.5/DA5 | 6 | I/O | General purpose I/O | (Open drain) |
| P5.6 | 7 | I/O | General purpose I/O |  |
| P5.7 | 8 | I/O | General purpose I/O |  |
| RST | 9 | I | High Active RESET |  |
| P3.0/RXD0 | 10 | I/O | Serial port 0 input |  |
| P3.1/TXD0 | 11 | I/O | Serial port 0 output |  |
| P3.2/INT0 | 12 | I/O | General purpose I/O /External interrupt(Standard 8051) |  |
| P3.3/INT1 | 13 | I/O | General purpose I/O /External interrupt(Standard 8051) |  |
| P3.4/T0 | 14 | I/O | General purpose I/O (Standard 8051) |  |
| P3.5/T1 | 15 | I/O | General purpose I/O (Standard 8051) |  |
| P7.6/CLKO | 16 | I/O | General purpose I/O /Clock out (CMOS) |  |
| P7.7 | 17 | I/O | General purpose I/O (CMOS) |  |
| X2 | 18 | O | Crystal Out |  |
| X1 | 19 | I | Crystal In |  |
| VSS | 20 | - | Ground |  |
| P6.0/AD0 | 21 | I/O | General purpose I/O (CMOS) /6-bit ADC channel 0 input |  |
| P6.1/AD1 | 22 | I/O | General purpose I/O (CMOS) /6-bit ADC channel 1 input |  |
| P6.2/AD2 | 23 | I/O | General purpose I/O (CMOS) /6-bit ADC channel 2 input |  |
| P6.3/AD3 | 24 | I/O | General purpose I/O (CMOS) /6-bit ADC channel 3 input |  |
| P6.4 | 25 | I/O | General purpose I/O (CMOS) |  |
| P6.5 | 26 | I/O | General purpose I/O (CMOS) |  |
| P6.6 | 27 | I/O | General purpose I/O (CMOS) |  |
| P6.7 | 28 | I/O | General purpose I/O (CMOS) |  |
| NC | 29 | - | No connection |  |
| NC | 30 | - | No connection |  |
| NC | 31 | - | No connection |  |
| P1.7 | 32 | I/O | General purpose I/O (Standard 8051) |  |
| P1.6 | 33 | I/O | General purpose I/O (Standard 8051) |  |
| P1.5 | 34 | I/O | General purpose I/O (Standard 8051) |  |
| P1.4 | 35 | I/O | General purpose I/O (Standard 8051) |  |
| P1.3 | 36 | I/O | General purpose I/O (Standard 8051) |  |
| P1.2 | 37 | I/O | General purpose I/O (Standard 8051) |  |
| P1.1 | 38 | I/O | General purpose I/O (Standard 8051) |  |
| P1.0 | 39 | I/O | General purpose I/O (Standard 8051) |  |
| VCC | 40 | - | 3.3V power |  |

  - U701 (Audio amplifier)TDA7053

| SYMBOL | PIN | DESCRIPTION |
| --- | --- | --- |
| n.c. | 1 | not connected |
| VC1 | 2 | DC volume control 1 |
| n.c. | 3 | not connected |
| VI (1) | 4 | voltage input 1 |
| VP | 5 | positive supply voltage |
| VI (2) | 6 | voltage input 2 |
| SGND | 7 | signal ground |
| VC2 | 8 | DC volume control 2 |
| OUT2+ | 9 | positive output 2 |
| PGND2 | 10 | power ground 2 |
| n.c. | 11 | not connected |
| OUT2- | 12 | negative output 2 |
| OUT1- | 13 | negative output 1 |
| PGND1 | 14 | power ground 1 |
| n.c. | 15 | not connected |
| OUT1+ | 16 | positive output 1 |

  - IC501 (OZ9910G, CCFL Inverter controller IC)

| Pin No. | Names | Description |
| --- | --- | --- |
| 1 | NDRV4 | Bottom MOSFET gate drive output in dual forward converter |
| 2 | PGND | High-current power ground |
| 3 | NDRV1 | Bottom MOSFET gate drive output in dual forward converter |
| 4 | GNDA | Low-current signal ground |
| 5 | CT | Timing capacitor of high frequency oscillator |
| 6 | LCT | Timing capacitor to set LPWM frequency |
| 7 | DIM | Control command input -- DC |
| 8 | ENA | Enable input |
| 9 | VSEN | Voltage sense feedback |
| 10 | CMP_SST | Soft start and loop compensation capacitor |
| 11 | ISEN | Current sense feedback |
| 12 | V5000 | Reference voltage output |
| 13 | VIN | Supply voltage for IC |
| 14 | HSB | High side driver buffer output |
| 15 | PDRV1 | Top MOSFET gate drive output in dual forward converter |
| 16 | PDRV2 | Top MOSFET gate drive output in dual forward converter |

# Chapter 4- Disassembly & Assembly

## 1. Exploded Diagram

2.LE1733 DISASSEMBLY BLOCK

ASSY,BEZEL FRONT, LE1733

1. BEZEL FRONT*1

2. Screw,P Cross,T4*16mm,Zn-Black*4

ASSY, BASE,LE1733

- BASE,LE1733*1

- Plate Base LE1733*1

- Rubber Foot,

L14.8mm*W9.6mm*T3.5mm*4

- Hinge,Right,LE1733*1

- Hinge,Left,LE1733*1

- Screw,P Cross,T5*12mm,Ni*4

- Screw,P Cross,T4*10mm,Ni*6

ASSY PANEL

- PANEL INNOLUX*1

- CHASSIS LE1708*1

- ASSY PCB*1

- LVDS CABLE*1

- INSULATOR,MYLAR*1

- Cable for grounding*1

- SPACER,SUPPORT*1

- FOIL,AL,DOUBLE COND*4

- BOLT,#4-40*11.8Ni*2

- SCREW,C,CROSS,M3*5,Ni*6

- INSULATOR,MYLAR,L29xW11.5mm,LE1708

ASSY,BACK COVER

1.  Cover Back*1

- Support Vesa LE1708*1

- LENS BACK LE1733*1

- SCREW,C,CROSS,M3*5,Zn-BLK*4

- ATOR,Mylar,L9.5 x W7 x T0.5mm,LE1720,RoHS

NOTES: The screw marked by blue color is the fix screw for two adjacent block

2. LE1733 ASSEMBLY BLOCK

ASSY, PANEL

- PANEL INNOLUX*1

- CHASSIS LE1708*1

- ASSY PCB*1

- LVDS CABLE*1

- INSULATOR,MYLAR*1

- Cable for grounding*1

- SPACER,SUPPORT*1

- FOIL,AL,DOUBLE COND*4

- BOLT,#4-40*11.8Ni*2

- SCREW,C,CROSS,M3*5,Ni*6

- SCREW,C,CROSS,M3*5,

Zn-BLK*4                             22.INSULATOR,MYLAR,L29xW11.5mm,LE1708

ASSY,BACK COVER

- Cover Back*1

- Support Vesa LE1708*1

- LENS BACK LE1733*1

- ATOR,Mylar,L9.5 x W7 x T0.5mm,LE1720,RoHS

ASSY, BASE,LE1733

- BASE,LE1733*1

- Plate base LE1733*1

- Rubber Foot,

L14.8mm*W9.6mm*T3.5mm*4

- Hinge,Right,LE1733*1

- Hinge,Left,LE1733*1

- Screw,P Cross,T5*12mm,Ni*4

- Screw,P Cross,T4*10mm,Ni*6

ASSY, BEZEL

1. BEZEL FRONT*1

2. Screw,P Cross,T4*16mm,Zn-Black*4

NOTES: The screw marked by blue color is the fix screw for two adjacent block

# Chapter 5		Test and adjustment

## 1.  Function key Definitions

1.1 Control buttons on the Back bezel

| CONTROL KEY | KEYS FUNCTION |
| --- | --- |
| [MENU/POWER] | A: By pressing [POWER] key, Show the OSD main Menu When Power is ON. B: Access to next menu page When OSD showed. C: Keep Pressing [POWER] key for about 2 second to turn off the monitor. D: Turn On the monitor. When Power is Off. |
| [+], | A. When “MENU OSD” showed ,Press this key to move to selected item or increase the value of some Submenu Items; B. When “MENU OSD” un-showed, press this key to show the Volume Menu. |
| [-] | A. When “MENU OSD” showed ,Press this key to move to selected the item or decrease the value of some Items; B. When “MENU OSD” un-showed, press the key to show the Volume Menu. |

Remark: Power off Monitor Method: Keep Pressing Power/Menu key for approximately 2 Seconds when there is signal input or it will turn off the monitor quickly under the condition of Power saving.

1.2 Hot Key Operation

| FUNCTION | DESCRIPTION |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | + | - | Menu/Power | POWER |  |
| Volume Menu | ● |  |  | On | Enable Volume menu. |
| Volume Menu |  | ● |  | ON | Enable Volume menu.. |
| Auto Adjustment | ● | ● |  | ON | Do auto adjustment |
| FACTORY MODE | ● | ● | ● | OFF | Enter Factory menu |

## 2.  OSD Control

| Menu | Item | Control Range | Default Value |
| --- | --- | --- | --- |
| Main Menu | Auto Adjustment | ------- | set sharpness to 50 |
|  | Brightness | 0 to 100 | 100 |
|  | Contrast | 0 to 100 | 50 |
|  | Color Temperature | 9300 K | ------------ |
|  |  | 6500 K | ------------ |
|  |  | User | ------------ |
|  | R-Contrast | 0 to 100 | 50 |
|  | G-Contrast | 0 to 100 | 50 |
|  | B-Contrast | 0 to 100 | 50 |
|  | Sharpness | 0 to 100 | 50 |
|  | Horizontal Position | 0 to 100 | Depend on each timing |
|  | Vertical Position | 0 to 100 | Depend on each timing |
|  | Phase | 0 to 100 | Depend on each timing |
|  | Clock | 0 to 100 | Depend on each timing |
|  | Power Saving Mode | 2 Kinds | Standard |
|  | OSD Position | 5 Kinds | Down Right |
|  | Language |  | ENGLISH |
|  | Recall | ------- | OSD language& position Not be load to default |
|  | Exit | ------- |  |
| Volume Menu | Volume | 0 to 100 | 60 |

## 3.  Factory Mode Introduction

When signal input, press “power key” to turn off the monitor. Press “-” , “+”and “Power” together to turn on the monitor. After power on, press “Power” to call out Main Menu, then press “-“for select the “F” item, then press “power/menu”, you can go into Factory mode.

fig1.                                  fig2.  Factory mode

AUTO COLOR:	Automatically calibrate chip ADC parameter by using chip internal DAC.

CONTRAST:		Contrast value

BACKLIGHT:    Brightness value

OFFSET: 		ADC offset value

GAIN: 			ADC Gain value

9300K:			Set color temperature 9300K

6500K:			Set color temperature 6500K

BURNIN:    	Turn on the Burning mode if choose On, Turn off burning mode if choose Off.

EXIT:         	Escape from Factory menu.

## 4.  Burn-in pattern

Enter the factory menu, Switch the BURN-IN ON, Plug out VGA, Plug out POWER Cable and then Plug in POWER Cable, Burn-in pattern will self generate automatically. You can not escape from Burn-in pattern until plug in VGA Cable or press the Menu/Power key,.

## 5.  Auto Color Balance (Automatically calibrate chip ADC parameter by using chip

internal DAC.)

If it is a new-built set and it is first time to do the “auto color”, please confirm the following steps:

-Connect the VGA cable with the standard video pattern generator and display 32-gray pattern on the monitor.

- Press “Power” to power off the monitor.

- Enter the Factory Mode.

- Then press the “Menu/POWER” to execute Auto Color item.

- After the “Auto Color” process finished, please press “Power” to restart monitor.

## 6.  EDID (Rewrite EDID data to EEPROM)

If we need to rewrite the EEPROM data, please confirm the following steps.

1. Plug in VGA Cable; we can rewrite the EDID data to EEPROM by using “EDID Rewrite” program.

2. If the “EDID Rewrite” process finished, please pull out AC power cable or press power key to restart

## 7.  Upload firmware to MCU via VGA Cable

Parallel Port

D-SUB 15PIN

7.1 Connect IPS board between monitor and PC as below configure

Insert to Parallel Port  on PC

ISP Board

LCD Monitor D-Sub

7.2. Select the MCU Type in ISP.exe which used in this monitor. Choose the corresponding firmware version, and load to MCU.

7.3. After finish, please plug out power cable and re-start monitor again.

## 8.  After repair, to ensure the quality you should do the following test and adjustment

| Item | Content |  |  |  | Equipment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test OSD function | 1.Signal is set as 1280×1024@60Hz under General-1 2.Checking whether each single function key and compound function key can be worked. |  |  |  | Chroma Signal Generator |  |
| Contrast Check | 1. Set input mode to 1280×1024@60Hz 2. Set Pattern to 32 gray shades 3. Set contrast to the max. The brightest 3~4 shades brightness cannot be distinguished. |  |  |  |  | Chroma Signal Generator |
| Color Temperature | 1. Do “Auto color Balance” at 1280×1024@60Hz, 32gray shades 2. Measure color temperature, check it complies with the following temperature : 6500K x = 0.313 +/- 0.03, y = 0.329+/-0.03 9300K x = 0.283 +/- 0.03, y = 0.298+/-0.03 |  |  |  |  | Chroma Signal Generator and color analyzer |
| Modes switching check | 1. Use Chroma Pattern Generator to make sequence. VESA (640x480 800x600 1024x768 1280x1024), and power saving signal. 2. Confirm the above timing modes must be full screen and the picture must be normal. 3. LED is amber at power saving mode. |  |  |  | Chroma Signal Generator |  |
| VGA cable detector | When VGA cable is not plugged, the monitor will work in power saving mode. |  |  |  | Visual check Chroma Signal Generator |  |
| Panel Flicker check | 1. Mode:1280×1024@60Hz 2. Set Brightness& contrast to default value 3. Do “Auto Adjustment” 4. Shut down PC to check whether there’s glitter on the center of the picture. |  |  |  | Chroma signal generator & PC |  |
| Power saving | 1. Mode:1280×1024@60Hz 2. Pattern: full white 3. Brightness: Max. 4. Contrast: Default 5. Check power consumption | at each modes |  |  |  | Chroma signal generator |
|  | State | Power Consumption | LED color |  |  |  |
|  | Normal |  | green |  |  |  |
|  | Stand By |  | amber |  |  |  |
|  | Power Key Off |  | no |  |  |  |

# Chapter 6- TROUBLE SHOOTING

## 1. Inverter trouble shooting

No raster?

LED Green?

Yes

Yes

Yes

Backlight cannot be turned on.

No

Check power supply

Is there high-level voltage on pin8 of IC501?

No

Is Ok R526?

Check I/F board

Yes

R526 open

No

Yes

Is there instantaneously pulse wave on pin1, pin3, pin15 and pin16 of IC501 at the moment                        of restart?

No

Is Ok IC501?

Yes

U501, U502 fail

No

IC501 fail

Yes

Is ok T501, T502?

T501, T502 fail

Are connected rightly CN501, CN502, CN503 and CN504?

No

Connecting the output connector again

No

Check protected circuit Q501, Q502, Q503, Q504

Yes

Yes

END

Is there 12Vdc voltage on pin13 of IC501?

Yes

Yes

2. Power trouble shooting_1

No power

Check primary rectifier voltage

Check circuit if short

Check pin3 of IC802 voltage about 1V

Check pin2 of IC802 voltage about 2V

Check pin1 of IC802 voltage is 5.8V

END

Check IC802, C805,T801

Check F801,  P801,D801

Check R801,R805, R822, R823, R817

Check R803, R807, R824, R825

Check R812, R816, C818

3. Power trouble shooting_2

POWER output UNSTABLE

Check circuit if short

Check reference voltage

Check pinR of IC801 voltage about 2.5V

Check Vpin3-4 of IC801 about 6V

Check ZD801,ZD802,ZD803,D803,D805

Check Q801,Q802

Check R809,R814,IC801,R818

- Check R810,R8Cover Back*1

- Support Vesa LE1708*1

- LENS BACK LE1733*1

- ATOR,Mylar,L9.5 x W7 x T0.5mm,LE1720,RoHS

11

Check C815,D806,R812

Check R809,R814

Check Vbe of  801 below 0.3V

Check feedback circuit

Check Vpin1-2 of IC801 bout 1V

END

4. Power trouble shooting_3

Unstable power

Check sample

Circuit

Check R810, R811, R818

Check the R pin voltage of IC803

about 2.5V

Check  D806, C815 if short

Check pin1 of IC802 voltage   is 5.8V

Check pin3 of IC802 voltage    about    1V

END

Check the C pin voltage of IC803 if 3V

Change R810, R811, R818

Check R809, R808, R814,D809

Change D806, C815

Change R801, R805, R822,R823,R817

Change IC803

5. Black Screen

6. White Screen

7. Bad Screen

# Chapter 7- RECOMMENDED PART LIST

| Item | Catalogue | INL P/N | Description | Min Order Q'ty |
| --- | --- | --- | --- | --- |
| 1 | Panel | 631102071290R | LCD PANEL 17" MT170EN01-V7-G1,AM17000057 | 10 |
| 2 |  | 631102071300R | LCD PANEL 17" MT170EN01-V7-G3,AM17000057 | 10 |
| 3 |  | 631102071420R | LCD PANEL 17" MT170EN01-V7-G1,AM17000057 | 10 |
| 4 |  | 631102071430R | LCD PANEL 17" MT170EN01-V7-G3,AM17000057 | 10 |
| 5 |  | 631102071600R | LCD PANEL 17" MT170EN01-V7-G2,AM17000057 | 10 |
| 6 |  | 631102071610R | LCD PANEL 17" MT170EN01-V7-G2,AM17000057 | 10 |
| 7 |  | 631102071620R | LCD PANEL 17" MT170EN01-V7-G1,AM17000057 | 10 |
| 8 |  | 631102071630R | LCD PANEL 17" MT170EN01-V7-G3,AM17000057 | 10 |
| 9 |  | 631102071710R | LCD PANEL 17" MT170EN01-V7-G1,AM17000057 | 10 |
| 10 |  | 631102071720R | LCD PANEL 17" MT170EN01-V7-G3,AM17000057 | 10 |
| 11 |  | 631102071910R | LCD PANEL 17" MT170EN01-V7-G1,AM17000057 | 10 |
| 12 |  | 631102071920R | LCD PANEL 17" MT170EN01-V7-G2,AM17000057 | 10 |
| 13 |  | 631102071930R | LCD PANEL 17" MT170EN01-V7-G3,AM17000057 | 10 |
| 14 | PCBA | 790491202622R | PCBA,MAIN/B(V7,EMEA),W/SPK,LE1733-6C0(HI | 20 |
| 15 | Cover | 714050005700R | ASSY,BACK COVER, LE1733 | 50 |
| 16 |  | 714020005700R | ASSY,BASE, LE1733 | 50 |
| 17 |  | 501010206110R | BEZEL,FRONT(B),HIQ, LE1733 | 50 |
| 18 | Cable | 453010100100R | CABLE,D-SUB 15P MALE 6FT BLACK/BLUE,SZ40 | 50 |
| 19 |  | 453070800250R | PWR CORD 10A/125V BLK 6FT UL/CSA,SVT 18x | 50 |
| 20 |  | 453030300120R | CABLE,AUDIO 1P 6FT BLACK/GREEN CP03B06P0 | 50 |
| 21 |  | 618100100600R | SPEAKER 1W 8Ω 450mm,RED/BLACK W/CASE,YD | 50 |
| 22 |  | 430303000090R | HRN LVDS FFC 30P 136mmACCX30136KU28MY,Ro | 50 |
| 23 |  | 430300100190R | HRN ASS'Y 1P 157mm BLACK,UL100 7#20 RoHS | 50 |
| 24 | Accessory | 703000001910R | KIT,ACCESSORY,HIQ(US), LE1733 | 100 |
| 25 | Packing | 506020010910R | CARTON,HIQ, LE1733 | 100 |
| 26 |  | 506040005410R | CUSHION,EPS-R, LE1708,ROHS | 100 |
| 27 |  | 506040005400R | CUSHION,EPS-L, LE1708,ROHS | 100 |

Attachment1-schematic

# Attachment 3- PCB Layout

Silkscreen Bottom Layer

L2 Bottom Layer

Silkscreen Top Layer